1.

Open Wireshark, capture network traffic while you refresh the Flipkart homepage, and identify one packet each belonging to the Application, Transport, Internet, and Network Access layers by examining their headers.

- Application Layer (HTTP/HTTPS): Example: GET / HTTP/1.1 request to flipkart.com. Header shows HTTP protocol → belongs to Application.

- Transport Layer (TCP): Example: TCP segment with flags like SYN/ACK. Header shows Source Port, Destination Port, Sequence Number.

- Internet Layer (IP): Example: IPv4 packet with Source IP (your device) and Destination IP (Flipkart server). Header shows IP addresses and TTL.

- Network Access Layer (Ethernet/ARP): Example: Ethernet II frame with Source MAC and Destination MAC. Header shows physical addressing.

2.

Create a table comparing the OSI and TCP/IP models, listing all layers side-by-side, and for each layer, give a real-world example from any app you use (e.g., WhatsApp message, Zomato order).

| OSI Model Layer | TCP/IP Model Layer | Real-World Example (App) |
| --- | --- | --- |
| Application (7) | Application | WhatsApp message formatting |
| Presentation (6) | Application | Instagram image compression |
| Session (5) | Application | Zoom call session setup |
| Transport (4) | Transport | TCP ensuring WhatsApp message delivery |
| Network (3) | Internet | IP routing for Zomato order request |
| Data Link (2) | Network Access | Wi-Fi frame carrying BookMyShow ticket data |
| Physical (1) | Network Access | Electrical signals over Ethernet cable |

3.

Given a screenshot of a TCP segment header (provided by your trainer), label the fields that belong to the Transport layer and explain what each field does in 1-2 lines.<br><br><em><strong>Hint:</strong> Look for fields like Source Port, Destination Port, Sequence Number, etc.</em>

- Source Port: Identifies sending application.

- Destination Port: Identifies receiving application.

- Acknowledgment Number: Confirms receipt of data.

- Flags (SYN, ACK, FIN, etc.): Control connection setup/teardown.

- Window Size: Flow control (how much data can be sent).

- Checksum: Error detection for segment integrity.

4.

Draw a diagram showing how a message sent from your device to Instagram's server passes through each TCP/IP layer, and annotate what happens at each step using simple app-related terms (e.g., 'message split into packets', 'IP address added', etc.).

Message → Instagram server:

- Application Layer: WhatsApp/Instagram formats your text/image.

- Transport Layer: TCP splits into segments, adds ports.

- Internet Layer: IP adds source/destination IP addresses.

- Network Access Layer: Ethernet/Wi-Fi adds MAC addresses, frames data.

- Physical Layer: Bits sent as signals over Wi-Fi/cable.

5.

Pick any one packet you captured in Wireshark and write which TCP/IP layer it belongs to, how you identified it, and what real-world action in an app (like sending a WhatsApp message or booking a ticket on BookMyShow) it represents.

- Packet Chosen: HTTP GET request to Flipkart.

- TCP/IP Layer: Application Layer.

- Identification: Protocol column shows HTTP.

- Real-World Action: Represents you opening Flipkart’s homepage in your browser