1.

Draw a simple diagram showing the pin layout of an RJ-45 connector and label the color codes for the T568A and T568B Ethernet wiring standards.

An RJ-45 connector  has 8 pins and 8 wires. T568A and T568B are two wiring standards that specify the color order of those wires.

RJ-45 pin layout

View from the front of the plug, with gold contacts facing you and the clip on the opposite side. Pin 1 is on the left.

White-striped wires are shown with a colored stripe; solid wires use the corresponding solid color.

T568A and T568B color order

| Pin | T568A | T568B |
| --- | --- | --- |
| 1 | White/Green | White/Orange |
| 2 | Green | Orange |
| 3 | White/Orange | White/Green |
| 4 | Blue | Blue |
| 5 | White/Blue | White/Blue |
| 6 | Orange | Green |
| 7 | White/Brown | White/Brown |
| 8 | Brown | Brown |

2.

Create a table comparing the key specifications of Ethernet standards 10BASE-T, 100BASE-TX, and 1000BASE-T, including speed, cable type, maximum cable length, and connector type.

| Specification | 10BASE-T | 100BASE-TX | 1000BASE-T |
| --- | --- | --- | --- |
| Speed | 10 Mbps | 100 Mbps | 1,000 Mbps (1 Gbps) |
| Cable type | Cat3 or better twisted pair | Cat5 or better twisted pair | Cat5e or better twisted pair |
| Maximum cable length | 100 m | 100 m | 100 m |
| Connector | 8P8C (commonly RJ-45) | 8P8C (commonly RJ-45) | 8P8C (commonly RJ-45) |
| Wire pairs used | 2 pairs | 2 pairs | All 4 pairs |
| Common application | Older Ethernet networks | Older office LANs | Modern home and college LANs |

3.

Given a scenario where you need to connect two laptops directly using Ethernet, explain which cable (straight-through or crossover) you would use and why.<br><br><em><strong>Hint:</strong> Think about how transmit and receive pairs are wired in each cable type.</em>

A crossover cable swaps the transmit and receive pairs at opposite ends, allowing two older Ethernet devices of the same type to communicate directly.

In older Ethernet interfaces without Auto-MDI/MDIX, the transmit pins on one laptop must connect to the receive pins on the other, and vice versa..

4.

List three real-world devices in your home or college that use Ethernet connections, and for each, specify the likely Ethernet standard (e.g., 100BASE-TX, 1000BASE-T) they support.

| Device | Likely Ethernet Standard | Reason |
| --- | --- | --- |
| Home Wi-Fi router | 1000BASE-T | Gigabit LAN ports support fast internet and local file transfers. |
| College desktop computer | 1000BASE-T | Gigabit Ethernet is common for modern computers in college labs. |
| Network printer | 100BASE-TX or 1000BASE-T | Either standard can connect a printer to the college or office LAN. |