1.

List the key differences between IPv4 and IPv6 addressing by creating a two-column table with at least 4 points for each.

| IPv4 | IPv6 |
| --- | --- |
| 32-bit address | 128-bit address |
| Written in dotted decimal (e.g., 192.168.1.1) | Written in hexadecimal, colon-separated (e.g., 2001:db8::1) |
| Supports manual and DHCP configuration | Supports auto-configuration (SLAAC) and DHCPv6 |
| Uses broadcast communication | Uses multicast and anycast (no broadcast) |

2.

Given the following scenario: You have 4 devices (laptop, phone, smart TV, and printer) connected to your home WiFi. Assign a unique static IPv4 address to each device using the 192.168.10.0/24 network, and write down the IP, subnet mask, and default gateway for each device.

Network: 192.168.10.0/24

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.10.1

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| Laptop | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| Phone | 192.168.10.11 | 255.255.255.0 | 192.168.10.1 |
| Smart TV | 192.168.10.12 | 255.255.255.0 | 192.168.10.1 |
| Printer | 192.168.10.13 | 255.255.255.0 | 192.168.10.1 |

3.

Calculate the number of valid subnets and hosts per subnet if you use a /27 CIDR subnet mask on a 192.168.1.0 network. List the first three valid subnet address ranges (network address to broadcast address) and the usable host IPs in each.

- Subnet mask: /27 = 255.255.255.224

- Host bits = 5 → Usable hosts = 25-2=30

- Number of subnets = 23=8

First Three Valid Subnets

| Subnet | Network Address | Broadcast Address | Usable Host Range |
| --- | --- | --- | --- |
| 1 | 192.168.1.0 | 192.168.1.31 | 192.168.1.1 – 192.168.1.30 |
| 2 | 192.168.1.32 | 192.168.1.63 | 192.168.1.33 – 192.168.1.62 |
| 3 | 192.168.1.64 | 192.168.1.95 | 192.168.1.65 – 192.168.1.94 |

4.

You are given the IP address 10.0.5.17 with subnet mask 255.255.255.0. Determine the network address, broadcast address, and the range of valid host addresses for this subnet.<br><br><em><strong>Hint:</strong> Convert the subnet mask to CIDR notation and use bitwise AND for network address calculation.</em>

- IP: 10.0.5.17

- Subnet mask: 255.255.255.0 → CIDR /24

- Network address: 10.0.5.0 (bitwise AND)

- Broadcast address: 10.0.5.255

- Valid host range: 10.0.5.1 – 10.0.5.254

5.

Use an online subnet calculator tool (such as https://www.subnet-calculator.com/) to verify your answer for the previous task. Take a screenshot of your calculation and write one line explaining if your manual answer matched the tool's output.

Result: The tool confirmed Network = 10.0.5.0, Broadcast = 10.0.5.255, Usable range = 10.0.5.1 – 10.0.5.254.

My manual calculation matched perfectly with the tool’s output.