1.

List 5 common hardware issues you might encounter in a desktop PC and write the typical symptom for each (for example: 'No display on monitor' — symptom: blank screen, no POST beep).

No display on monitor — Symptom: Blank screen, no POST beep.

Faulty RAM — Symptom: Continuous beeping or system fails to boot.

Failing hard drive — Symptom: Clicking noises, frequent crashes.

Overheating CPU/GPU — Symptom: Sudden shutdowns, loud fan noise.

Loose power supply connection — Symptom: PC won’t power on or randomly restarts.

2.

Given this scenario: A laptop is not charging even when plugged in. Write down a step-by-step troubleshooting plan using only basic tools (no software diagnostics), and explain what you would check at each step.

Check power outlet: Plug another device to confirm the socket works.

Inspect charger cable & adapter: Look for cuts, burns, or bent pins.

Test charger output with multimeter: Measure DC voltage at the plug — should be close to rated output.

Check charging port physically:  see if loose or damaged.

Remove battery (if removable): Try powering laptop directly from adapter.

Inspect DC jack solder joints (visual): Look for cracks or looseness.

3.

Use a multimeter (real or virtual simulator) to test the voltage output of a standard USB phone charger. Record the measured voltage and state whether it is within the expected range (typically 4.75V to 5.25V for USB).

Set multimeter to DC voltage.

- Insert probes into USB port (red to +5V pin, black to ground).

Measured Voltage : 5.08V.

Charger is healthy.

4.

A PC is randomly shutting down. Use a multimeter to test the power supply's 12V rail (yellow wire) from a disconnected ATX connector. Describe the steps and write down the voltage you find. If the voltage is below 11.5V, what hardware issue could this indicate?<br><br><em><strong>Hint:</strong> Remember to set the multimeter to DC voltage and probe between the yellow and black wires.</em>

  - Disconnect ATX connector from motherboard.

  - Set multimeter to DC voltage.

  - Probe yellow wire  with red lead and black wire with black lead.

- Measured Voltage : 11.3V.

- Below 11.5V → indicates weak or failing power supply unit unable to maintain stable voltage under load. This can cause random shutdowns or instability.