1.

List the IP address classes (A, B, C, D, E) and give one real-world example of a device or service that might use each class. Write your answers in a table format.

| Class | IP Address Range | Default Subnet Mask | Purpose | Real-World Example |
| --- | --- | --- | --- | --- |
| A | 1.0.0.0 - 126.255.255.255 | 255.0.0.0 | Very large networks | A large multinational company network |
| B | 128.0.0.0 - 191.255.255.255 | 255.255.0.0 | Medium-sized networks | University or college campus network |
| C | 192.0.0.0 - 223.255.255.255 | 255.255.255.0 | Small networks | Home WiFi router (e.g., 192.168.1.1) |
| D | 224.0.0.0 - 239.255.255.255 | Not used for hosts | Multicast communication | Live video streaming service |
| E | 240.0.0.0 - 255.255.255.255 | Reserved | Experimental/Research | Network research testing |

2.

Given the IP address 192.168.10.45 with subnet mask 255.255.255.0, calculate the network address, broadcast address, and the range of valid host addresses.<br><br><em><strong>Hint:</strong> Show your working steps for each calculation.</em>

Step 1: Find the network address

The subnet mask 255.255.255.0 means the first three octets represent the network.

Network address = 192.168.10.0

Step 2: Find the broadcast address

Set all host bits to 1.

Broadcast address = 192.168.10.255

Step 3: Find the valid host range

- First valid host: 192.168.10.1

- Last valid host: 192.168.10.254

Step 4: Calculate the number of usable hosts

Host bits = (32-24=8)

Usable hosts = (2^8-2=254)

3.

Create a subnetting plan for a small food delivery startup like Zomato that needs 4 separate subnets: one for Delivery Partners, one for Restaurants, one for Customers, and one for Admins. Assign a unique subnet (with network address and subnet mask) for each group using the 10.0.0.0/24 network.

Given network: 10.0.0.0/24

We need four separate subnets. Borrow 2 bits from the host portion:

- Required subnets: 2^2=4

- New prefix: 24 + 2 =26

- New subnet mask: 255.255.255.192

- Addresses per subnet: 2^6=64

- Usable hosts per subnet: 64-2=62

| Group | Network Address | Subnet Mask | Valid Host Range | Broadcast Address |
| --- | --- | --- | --- | --- |
| Delivery Partners | 10.0.0.0/26 | 255.255.255.192 | 10.0.0.1–10.0.0.62 | 10.0.0.63 |
| Restaurants | 10.0.0.64/26 | 255.255.255.192 | 10.0.0.65–10.0.0.126 | 10.0.0.127 |
| Customers | 10.0.0.128/26 | 255.255.255.192 | 10.0.0.129–10.0.0.190 | 10.0.0.191 |
| Admins | 10.0.0.192/26 | 255.255.255.192 | 10.0.0.193–10.0.0.254 | 10.0.0.255 |

Four separate subnets, each supporting 62 usable host addresses.

4.

You are given the following scenario: A college WiFi network uses the IP range 172.16.0.0/16. The admin wants to create at least 20 subnets for different buildings. Calculate the new subnet mask and the number of hosts available per subnet.<br><br><em><strong>Constraint:</strong> Show your calculation steps and final answers.</em>

Given:

- Original network: 172.16.0.0/16

- Required: At least 20 subnets

Step 1: Calculate borrowed bits

- 2^4=16 — insufficient

- 2^5=32 — sufficient

Therefore, borrow 5 bits.

Step 2: Calculate the new prefix

New prefix = 16 + 5 = 21

Step 3: Find the new subnet mask

21 corresponds to:

255.255.248.0

Step 4: Calculate hosts per subnet

Remaining host bits = 32-21=11

Total addresses per subnet:

2^11=2048

Usable hosts:

2^11-2=2046

5.

A friend sends you a message asking why their device with IP 192.168.1.300 is not connecting to the network. Write a short explanation identifying the error and what a valid IP address in that subnet should look like.

An IPv4 address contains four octets separated by dots. Each octet must have a value between 0 and 255.

The address 192.168.1.300 is invalid because the last octet, 300, exceeds the maximum value of 255.

A valid address in the same /24 subnet could be 192.168.1.100, assuming that address is not already in use and is available for assignment.