1.

List all the main components you find inside a desktop PC after opening the cabinet, and write a short description (1-2 lines) for each.

- Motherboard: The main circuit board that connects all components.

- CPU (Processor): Executes instructions and performs calculations.

- RAM (Memory): Temporary storage for active programs and processes.

- GPU (Graphics Card): Handles rendering of images, videos, and games.

- HDD (Hard Disk Drive): Mechanical storage for large files, slower but high capacity.

- SSD (Solid-State Drive): Faster storage with no moving parts.

- Power Supply Unit (PSU): Converts AC power to DC and distributes it to components.

- Cooling System (Fans/Heatsinks): Keeps CPU/GPU temperatures under control.

- Cables (SATA, Power, Data): Connect storage drives and components to the motherboard.

2.

Take clear photos or draw labeled diagrams of each step as you disassemble a desktop PC, and organize them in a document with captions explaining what you did at each step.<br><br><em><strong>Hint:</strong> Include steps like unplugging power, removing side panel, disconnecting cables, and removing RAM, HDD/SSD, and motherboard.</em>

- Unplug Power Cable:

Caption: Disconnected the PSU power cable to ensure safety before opening the case.

- Remove Side Panel:

Caption: Unscrewed and slid off the side panel to access internal components.

- Disconnect Cables:

Caption: Detached SATA, power, and data cables to free components for removal.

- Remove GPU:

Caption: Unscrewed and released latch before carefully pulling out the graphics card.

- Remove RAM:

Caption: Pushed side clips outward and lifted RAM sticks from DIMM slots.

- Remove Storage Drives (HDD/SSD):

Caption: Unscrewed and slid out drives from their bays.

- Remove Motherboard:

Caption: Unscrewed motherboard from standoffs and lifted it out of the case.

3.

Reassemble the PC you disassembled, documenting each step with a brief explanation of what you connected or installed and why the order matters.

1. Install the Power Supply Unit (PSU)

- Action: Slide the PSU into its bay and secure with screws.

- Why first: It provides all power connections; installing it early ensures cables can be routed cleanly around other components.

2. Mounted the Motherboard

- Action: Place the motherboard on standoffs inside the case and screw it in.

- Why now: It’s the central hub; other components (CPU, RAM, GPU, drives) connect to it, so it must be secured before adding them.

3. Install the CPU and Cooler

- Action: Open the CPU socket latch, place CPU correctly (match triangle markers), apply thermal paste, and attach the cooler.

- Why here: CPU installation requires a stable motherboard; cooler must be mounted before other tall components block access.

4. Insert RAM Modules

- Action: Push RAM sticks into DIMM slots until clips lock.

- Why now: RAM slots are accessible before GPU or cables crowd the area.

5. Install the GPU (Graphics Card)

- Action: Insert GPU into PCIe slot, secure with screws, and connect power cables if required.

- Why after RAM: GPU is large and can block access to RAM slots if installed earlier.

6. Mount Storage Drives (HDD/SSD)

- Action: Screw drives into bays , then connect SATA/power cables.

- Why now: Storage is less bulky, so it’s easier to install after major components are in place.

7. Reconnect Cables

- Action: Attach PSU cables (24-pin ATX, CPU 8-pin, GPU PCIe, SATA power), data cables (SATA, front-panel connectors, USB headers).

- Why last: Cable management is easier once all components are fixed in place.

8. Close the Case

- Action: Replace side panel and secure with screws.

- Why final step: Protects components and prepares the PC for testing.

9. Reconnect Power & Test Boot

- Action: Plug in PSU to mains, press power button, and check BIOS/POST screen.

- Why important: Confirms all components are detected and functioning before OS loads.

4.

Create a checklist of safety precautions and tools needed before starting the PC assembly/disassembly process, and explain why each is important.

Safety Precautions & Why They Matter

- Unplug the PC: Stops live electricity from flowing. This keeps you safe from shocks and stops parts from breaking.

- Wear a wrist strap: Stops static electricity from your body. Static can ruin tiny computer chips instantly.

- Touch bare metal: Drains extra static from your hands before you touch parts.

- Keep food and drinks away: Stops spills. Liquids will ruin electronics permanently.

Tools Needed & Why They Matter

- Magnetic Phillips screwdriver: Tightens and loosens screws. The magnet stops small screws from falling inside the PC.

- Small flashlight: Helps you see dark corners. You need to see tiny cable plugs clearly.

- Small bowls or cups: Holds loose screws. This stops you from losing important parts.